Enzymatic Assay of PHOSPHATASE, ALKALINE1

(EC 3.1.3.1)

Diethanolamine Assay

PRINCIPLE:

p-Nitrophenyl Phosphate + H2O  p-Nitrophenol + Pi

Abbreviation used:

Pi = Inorganic Phosphate

CONDITIONS: T = 37°C, pH = 9,8, A405nm, Light path = 1 cm

METHOD: Continuous Spectrophotometric Rate Determination

REAGENTS:

- 1,0 M Diethanolamine Buffer with 0,50 mM Magnesium Chloride, pH 9,8 at 37°C
(Prepare 50 ml using Diethanolamine and Magnesium Chloride, Hexahydrate. Dissolve the Magnesium Chloride complete in deionized water before adding the Diethanolamine. Adjust to pH 9,8 at 37°C with 5 M HCl.
PREPARE FRESH.)

- 150 mM p-Nitrophenyl Phosphate Solution (PNPP)
(Prepare 2 ml in deionized water 
PREPARE FRESH.)

- Phosphatase, Alkaline Enzyme Solution
(Immediately before use, prepare a solution containing 0,1 – 0,2 unit/ml of Alkaline
Phosphatase in cold Reagent A.)

PROCEDURE:

Pipette (in milliliters) the following reagents into suitable cuvettes:

|  | Test | Blank |
| --- | --- | --- |
| Reagent A (Buffer) | 2,70 | 2,80 |
| Reagent B (PNPP) | 0,30 | 0,30 |

Mix by inversion and equilibrate to 37°C. Monitor the A405nm until constant, using a suitably
thermostatted spectrophotometer. Then add:

|  | Test | Blank |
| --- | --- | --- |
| Reagent C (Enzyme Solution) | 0,10 | ------ |

Immediately mix by inversion and record the increase in A405nm for approximately 10 minutes.
Obtain the DA405nm/minute using the maximum linear rate for both the Test and Blank.

CALCULATIONS:

Units/ml enzyme =(ΔA405nm/min Test - ΔA405nm/min Blank)(3,1)(df)(18,5)(0,1)

3,1 = Volume (in milliliters) of assay

df = Dilution factor

18,5 = Millimolar extinction coefficient of p-Nitrophenol at 405 nm

0,1 = Volume (in milliliters) of enzyme used

Units/mg solid = units/ml enzymemg solid/ml enzyme

Units/mg protein = units/ml enzymemg protein/ml enzyme

UNIT DEFINITION:

One unit will hydrolyze 1,0 mmole of p-nitrophenyl phosphate per minute at pH 9,8 at 37°C.

FINAL ASSAY CONCENTRATIONS:

In a 3,10 ml reaction mix, the final concentrations are 903 mM diethanolamine, 0,45 mM magnesium chloride, 14 mM p-nitrophenyl phosphate and 0,01 – 0,02 unit alkaline phosphatase.

REFERENCES:

Walter, K. and Schütt, C. (1974) in Methods of Enzymatic Analysis (Bergmeyer, H.U. ed) 2nd ed.,

Volume II, pp 860-864, Academic Press, Inc., NY