Enzymatic Assay of RIBOKINASE

(EC 2.7.1.15)

PRINCIPLE:

ATP + D-Ribose  ADP + D-Ribose 5-P

D-Ribose 5-P  D-Ribulose 5-P

D-Ribulose 5-P  > D-Xylulose 5-P

D-Xylulose 5-P + D-Ribose 5-P                                    GAP + Sedoheptulose 7-P

GAP  DHAP

DHAP + β-NADH β-NAD + a-Glycerophosphate

Abbreviations used:

ADP = Adenosine 5'-Phosphate
PRI= Phosphoriboisomerase
Ru-5-P-3-Epim = Ribulose-5-Phosphate-3-Epimerase
TK = Transketolase
GAP = Glyceraldehyde 3-Phosphate
TPI = Triosephosphate Isomerase
DHAP = Dihydroxyacetone Phosphate
ß-NADH = ß-Nicotinamide Adenine Dinucleotide, Reduced Form
a-GDH = a-Glycerophosphate Dehydrogenase
ß-NAD = ß-Nicotinamide Adenine Dinucleotide, Oxidized Form

CONDITIONS: T = 37°C, pH = 7,7, A340nm, Light path = 1 cm

METHOD: Continuous Spectrophotometric Rate Determination

REAGENT:

- 250 mM Glycylglycine Buffer, pH 7,7 at 37°C
(Prepare 100 ml in deionized water using Glycylglycine. Adjust to pH 7,7 at 37°C with 1 M NaOH.)

- 100 mM Ribose Solution (Ribose)
(Prepare 1 ml in deionized water using D(-)Ribose.)

- 2 mM Cocarboxylase Solution (Cocarboxylase)
(Prepare 1 ml in deionized water using Cocarboxylase.)

- 2,5 mM β-Nicotinamide Adenine Dinucleotide Solution, Reduced Form (β-NADH)
(Dissolve the contents of one 5 mg vial of β-Nicotinamide Adenine Dinucleotide, Reduced Form, Disodium Salt in the appropriate volume of deionized water.)

- 300 mM Magnesium Chloride Solution (MgCl2)
(Prepare 1 ml in deionized water using Magnesium Chloride, Hexahydrate.)

- 180 mM Adenosine 5'-Triphosphate Solution (ATP)
(Prepare 1 ml in deionized water using Adenosine 5'-Triphosphate, Disodium Salt.)

- Ribulose-5-Phosphate-3-Epimerase Enzyme Solution
(Ru-5-P-3-Epim) (Immediately before use, prepare a solution containing 10 units/ml of D-Ribulose 5-Phosphate-3-Epimerase.)

- Transketolase Enzyme Solution (TK)
(Immediately before use, prepare a solution containing 10 units/ml of Transketolase in cold deionized water.)

- a-Glycerophosphate Dehydrogenase/Triosephosphate Isomerase Enzyme Solution (a-GDH/TPI)
(Immediately before use, prepare a solution containing 100 units/ml of a-Glycerophosphate Dehydrogenase-Triosephosphate Isomerase in cold deionized water. The 100 units/ml is based on
a-GDH units.)

- Phosphoriboisomerase Enzyme Solution (PRI)
(Immediately before use, prepare a solution containing 50 units/ml of Phosphoriboisomerase in cold deionized water.)

- Ribokinase Enzyme Solution (Ribokinase)
(Immediately before use, prepare a solution containing 0,3 – 0,5 unit/ml of Ribokinase in cold Reagent A.)

PROCEDURE:

Pipette (in milliliters) the following reagents into suitable cuvettes:

|  | Test | Blank |
| --- | --- | --- |
| Reagent A (Buffer) | 2,25 | 2,35 |
| Reagent B (Ribose) | 0,10 | 0,10 |
| Reagent C (Cocarboxylase) | 0,05 | 0,05 |
| Reagent D (ß-NADH) | 0,15 | 0,15 |
| Reagent E (MgCl2) | 0,10 | 0,10 |
| Reagent F (ATP) | 0,05 | 0,05 |
| Reagent G (Ru-5-P-3-Epim) | 0,05 | 0,05 |
| Reagent H (TK) | 0,05 | 0,05 |
| Reagent I (a-GDH/TPI) | 0,05 | 0,05 |
| Reagent J (PRI) | 0,05 | 0,05 |

Mix by inversion and equilibrate to 37°C. Monitor the A340nm until constant, using a suitably thermostatted spectrophotometer. Then add:

| Reagent K (Ribokinase) | 0.10 | ------ |
| --- | --- | --- |

Immediately mix by inversion and record the decrease in A340nm for approximately 15 minutes. The maximum linear rate usually occurs between 10 - 15 minutes. Obtain the ΔA340nm/min using the maximum linear rate for both the Test and Blank.

CALCULATIONS:

Units/ml enzyme = (ΔA340nm/min Test - ΔA340nm/min Blank)(3)(df)(6,22)(0,1)

3 = Total volume (in milliliters) of assay

df = Dilution factor

6,22 = Millimolar extinction coefficient of ß-NADH at 340nm

0,1 = Volume (in milliliter) of enzyme used

Units/mg protein = units/ml enzymemg protein/ml enzyme

UNIT DEFINITION:

One unit will convert 1,0 μmole of D-ribose to D-ribose 5-phosphate per minute at pH 7,7 at 37°C in the presence of ATP and magnesium.

FINAL ASSAY CONCENTRATION:

In a 3,00 ml reaction mix, the final concentrations are 196 mM glycylglycine, 3,3 mM D-ribose, 0,03 mM cocarboxylase, 0,13 mM ß-nicotinamide adenine dinucleotide, reduced form, 10 mM magnesium chloride, 3 mM adenosine 5'-triphosphate, 0,5 unit ribulose-5-phosphate-3-epimerase, 0,5 unit transketolase, 5 units a-glycerophosphate dehydrogenase, approximately 50 units triosephosphate isomerase, 2,5 units phosphoriboisomerase, and 0,03 – 0,05 unit ribokinase.

REFERENCE:

Agranoff, B.W. and Brady, R.O. (1956) Journal of Biological Chemistry 219, 221-229

Racker, E. (1974) in Methods of Enzymatic Analysis (Bergmeyer, H.U. ed) 2nd ed., Volume III, 1342-1345, Academic Press, New York, NY

NOTES:

1. Phosphoriboisomerase Unit Definition: One unit will convert 1,0 μmole of D-ribose 5-phosphate to D-ribulose 5-phosphate per minute at pH 7,7 at 30°C.

2. D-Ribulose-5-Phosphate-3-Epimerase Unit Definition: One unit will convert 1 μmole of D-ribulose 5- phosphate to D-xylulose 5-phosphate per minute at pH 7,7 at 25°C in a coupled system with ribose-5-phosphate, ß-NADH, transketolase, a-glycerophosphate dehydrogenase/triosephosphate isomerase and cocarboxylase.

3. Transketolase Unit Definition: One unit will produce 1 μmole of glyceraldehyde 3-phosphate from xylulose 5-phosphate per minute at pH 7,7 at 25°C in the presence of ribose 5-phosphate, cocarboxylase, and magnesium in a coupled system with triosephosphate isomerase and
a-glycerophosphate dehydrogenase.

4. a-Glycerophosphate Dehydrogenase Unit Definition: One unit will convert 1,0 μmole of dihydroxyacetone phosphate to a-glycerophosphate per minute at pH 7,4 and 25°C.

5. Triosephosphate Isomerase Unit Definition: One unit will convert 1,0 μmole of D-glyceraldehyde 3-phosphate to dihydroxyacetone phosphate per minute at pH 7,6 and 25°C.

6. This assay is based on the cited references.